Energy Cost Optimization in Cooling Systems Using Deep Reinforcement Learning

Abstract — Cooling systems consume a major portion of building electricity. This paper proposes a Deep Q-Network (DQN) reinforcement learning agent that controls a cooling system intelligently to minimise energy cost while maintaining occupant comfort. The agent learns to avoid peak tariff hours, pre-cool spaces before expensive periods, and shut off during unoccupied times — all automatically through trial-and-error learning. Results show a 47.9% cost reduction over a standard thermostat baseline.

Keywords — reinforcement learning, DQN, energy optimization, cooling system, HVAC, reward shaping.

I. INTRODUCTION

Cooling systems such as air conditioners and HVAC units account for up to 40% of total building energy consumption. Conventional thermostats operate on fixed rules — they turn ON when temperature is high and OFF when it is low — without any awareness of electricity prices, room occupancy, or weather conditions. This leads to significant energy waste and high electricity bills.

This paper addresses this problem by replacing the fixed-rule controller with a Reinforcement Learning (RL) agent. The agent learns through experience which cooling actions save the most money while keeping the room comfortable — adapting its behaviour to time-of-use tariffs, occupancy patterns, and outdoor conditions automatically.

II. PROBLEM FORMULATION

A. What the Agent Controls

The agent controls a single-zone cooling system (air conditioner). At every 15-minute time step, it decides how much cooling to apply based on the current state of the environment.

B. State — What the Agent Observes

s_t = (T_in,  T_out,  P_e,  Occupancy,  E_prev,  Hour)

T_in = current indoor temperature,  T_out = outdoor temperature,  P_e = electricity tariff (peak / off-peak),  Occupancy = is the room occupied?,  E_prev = energy used last step,  Hour = current hour of the day.

C. Action — What the Agent Can Do

- Action 0: Turn cooling OFF

- Action 1: Run cooling at LOW power

- Action 2: Run cooling at HIGH power

D. Reward and Penalty

TABLE I.   Reward and Penalty Signal Design

| Condition | Signal | Value |
| --- | --- | --- |
| Temperature within comfort range (occupied) | Reward | +10 |
| Cooling during off-peak tariff hours | Reward | +5 |
| System OFF during unoccupied period | Reward | +8 |
| Energy below daily budget | Reward | +6 |
| Temperature above comfort range | Penalty | -15 |
| Overcooling (temperature too low) | Penalty | -8 |
| Full power cooling during peak tariff | Penalty | -12 |
| Cooling an unoccupied room | Penalty | -10 |
| Frequent ON/OFF switching | Penalty | -7 |
| Daily energy budget exceeded | Penalty | -20 |

E. Reward Equation

R(s,a) = R_cost  +  R_comfort  +  R_efficiency

R_cost     = -P_e * E(a)            [2x penalty during peak hours]

R_comfort  = -alpha * |T_in - T_setpoint|  if outside comfort range

R_efficiency = +beta  if unoccupied & OFF   -gamma if frequent switching

III. METHODOLOGY

The proposed methodology follows an 8-step training pipeline. At each time step the agent observes the environment state, selects a cooling action using an epsilon-greedy policy, receives a reward signal, and stores the transition in a replay buffer. The Q-network is updated from random mini-batches, with a separate target network synchronised every 1,000 steps to stabilise learning. Training runs for 1,000 episodes before evaluation. Figure 1 illustrates the complete methodology flowchart.

Figure 1. Methodology flowchart — 8-step RL training pipeline

IV. SYSTEM ARCHITECTURE

The system comprises six modules operating in a closed loop: the environment, perception module, Q-network, action execution, reward module, replay buffer, and training/target network pair. At every 15-minute step, the environment produces a state vector that flows through the agent and back as a cooling action. Figure 2 shows the full architecture diagram.

Figure 2. System architecture — closed-loop RL cooling control

V. RESULTS

TABLE II.   Performance Comparison

| Method | Daily Cost | Comfort Violations | Peak Usage |
| --- | --- | --- | --- |
| Fixed Thermostat | 142.5 INR | 3.1% | 18.4 kWh |
| Time-Scheduled | 98.7 INR | 12.4% | 4.2 kWh |
| Proposed DQN Agent | 74.3 INR | 4.8% | 3.1 kWh |

The DQN agent achieves the lowest daily cost (74.3 INR) — a 47.9% saving over the fixed thermostat. Comfort violations remain low at 4.8%, far better than the time-scheduled approach (12.4%). Peak electricity usage is also the lowest at 3.1 kWh.

VI. WHAT THE AGENT GAINS

- Cost Reduction: 47.9% lower electricity bill by avoiding peak tariff hours.

- Pre-Cooling Strategy: The agent learns to cool the room before peak hours using cheap electricity, then coast through the expensive period.

- Occupancy Awareness: Automatically stops cooling when the room is empty.

- No Overcooling: Learns the minimum cooling needed — not more, not less.

- Equipment Protection: Smoother operation from penalising frequent switching.

- Weather Adaptation: Adjusts strategy based on outdoor temperature each day.

- Fully Autonomous: No human intervention needed once trained.

VII. CONCLUSION

This paper presented a Deep Q-Network agent for energy cost optimization in cooling systems. By learning from a reward function that jointly penalises cost and comfort violations, the agent autonomously discovers intelligent control strategies — including pre-cooling, occupancy-based shutdown, and peak-hour avoidance — that significantly outperform conventional rule-based controllers. The proposed approach achieves a 47.9% reduction in daily energy cost while maintaining acceptable thermal comfort, demonstrating the practical value of reinforcement learning for smart building energy management.